### AlphaServer 1000 Pedestal–Rackmount–Cabinet 
V96-2.7—4 Apr 1997
DIGITAL Systems and Options Catalog

Product Description 	The AlphaServer 1000 5/300 system is a 300 MHz Alpha microprocessor server, with 2 MB ECC cache. For investment protection, customers get a choice of three popular operating systems: DIGITAL UNIX, OpenVMS, and Microsoft Windows NT Server.  Integrated on system motherboard are SVGA controller, diskette controller, two serial ports, one parallel port, and keyboard and mouse controller.

Advanced server management features are provided with all AlphaServer 1000 shipments via the bundled ServerWORKS Manager kit.  The kit  provides remote management capability through Simple Network Management  Protocol agents (SNMP) for Windows NT, DIGITAL UNIX and OpenVMS.  The  management console software runs on any Windows NT or Windows 95 PC.  The SNMP agents required to manage the server ship with the TCP/IP services  of the operating system (with the exception of Windows NT which ships with  the ServerWORKS kit).  The SNMP agents collect critical server information  including system status, I/O information, storage and disk information and  network status information.  Thresholds can be set to monitor key server events and alarms are forwarded to the ServerWORKS console.  ServerWORKS  Manager V2.0 includes remote pager support for alarm notification.  In addition to ServerWORKS, DIGITAL includes key third party software applications including DIGITAL Power Management Software for Alpha provided  by EXIDE, and application / database management software through BMC software's PATROL family of products.

AlphaServer 1000 5/300 Pedestal System offers up to 10 internal storage devices including a floppy diskette drive, a CD-ROM drive and provision for an additional removable media device.  Pedestal systems support up to 1 GB of memory and over 30 GB of internal storage with seven 4.3 GB narrow StorageWorks disk drives.  The AlphaServer 1000 pedestal enclosure offers high-reliability features such as fully redundant power supplies, internal RAID, hot swap disk, dual Fast Narrow/Wide SCSI backplane, and ECC memory.

AlphaServer 1000 5/300 Rackmount and Cabinet Systems offer up to six internal storage devices including a floppy diskette drive, a CD-ROM drive and provision for an additional removable media device.  Rackmount systems support up to 1 GB of memory and over 12.9 GB of internal storage with three 4.3 GB narrow disk drives.  Rackmount systems support additional storage through four SCSI expansion ports located on rear panel of system for connecting StorageWorks shelves, each external expansion port requires a SCSI controller. Rackmount and Cabinet systems do not offer redundant power supplies or hot swap internal disk support, hot swap disk are supported in rackmount StorageWorks shelves.

DIGITAL believes the information in this publication is accurate as of its publication date; such information is subject to change without notice. DIGITAL is not responsible for any inadvertent errors.

DIGITAL conducts its business in a manner that conserves the environment and protects the safety and health of its employees, customers, and the community.

DIGITAL, the DIGITAL logo are trademarks of Digital Equipment Corporation.

Printed in USA. Copyright 1997 Digital Equipment Corporation. All rights reserved.

### Step 1—AlphaServer 1000 Pedestal Systems

- Windows NT systems include Windows NT Server plus 5-client access license, media (CD-ROM) kit North American English.

- DIGITAL UNIX and OpenVMS Packaged systems and Base systems ordered with a minimum of a 1.05 GB disk, include factory installed software.

- Uninterruptable Power Supplies are available; see UPS Information following System Specifications.

- Options ordered that are factory installable, will be factory installed unless specified as spares.

Pedestal Systems include

- Alpha microprocessor 21164 300-MHz CPU with 2 MB ECC onboard cache

- Pedestal enclosure which includes:

- 10 expansion slots: Seven EISA slots, two PCI slots and one PCI/EISA combination slot

- Integrated Fast Narrow SCSI-2 controller with DMA and external SCSI-2 connector

- Integrated SVGA graphics controller

- 20 SIMM memory slots support 4 memory options

- 10 storage slots:
One diskette drive slot
One CD-ROM drive slot
One additional removable media slot
Seven StorageWorks hard drive slots

- 450-Watt self-sensing, self-switching (120/240V, 60/50Hz) power supply

- Two serial ports, support full duplex asynchronous modem control

- One bi-directional enhanced parallel port

- PS/2 style keyboard port and mouse port

- 1.44 MB diskette drive (RX23L) in dedicated slot

- 3-button mouse

- North American variants include 120V power cord (North America, Japan).  Mandatory selection of country-specific power cord for all non-North American variants, see Step 10.

- English language documentation.  Certain language selections are available

- EISA Configuration Utility (ECU)

- Integrated Server Management

- Hardware Warranty

- Three-year on-site*

- Software Warranty: 90-day SPD conformance with advisory telephone support*

- Windows NT Server plus 5-client access license, media (CD-ROM) kit, or

- DIGITAL UNIX 2-user base license, Internet AlphaServer System Software Kit, DIGITAL NAS Base Server 200 license and or

- OpenVMS base license with System Manager license and DIGITAL NAS 200 license

Pedestal Packaged Systems include

- PCI-based Ethernet—uses one PCI slot.

- 600 MB CD-ROM—uses one removable media slot.

- One hard drive, see below

- One memory option, see below

- North American variants include a 101-key, PC style North American English keyboard.  Select country-specific keyboard for non-North American variants, see Step 10.

*	Service upgrades are available; see Step 12, Hardware and Software Supplemental Services.

| 5/300 Pedestal Packaged Systems |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Hard Drive | Monitor |
| PB75B-AA | Windows NT | Included* | 120 V | Included | 64 MB | 2.1 GB | Required |
| PB75B-AB | Windows NT | Mandatory | Mandatory | Required | 64 MB | 2.1 GB | Required |
| PB75B-FA | DIGITAL UNIX | FIS | Included | Included | 64 MB | 2.1 GB | Recommended |
| PB75B-FB | DIGITAL UNIX | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Recommended |
| PB75B-MA | OpenVMS | FIS | Included | Included | 64 MB | 2.1 GB | Recommended |
| PB75B-MB | OpenVMS | FIS | Mandatory | Recommended | 64 MB | 2.1 GB | Recommended |

*	Windows NT Server license, media (CD-ROM) kit North American English.

KEY:	Mandatory items must be on purchase order at initial order acceptance

Required items are essential for full system operation.

Recommended items enhance system functionality

FIS = Factory Installed Software

### Step 1—AlphaServer 1000 Pedestal Systems (continued)

Pedestal Base systems

| 5/300 Pedestal Base systems |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Hard Drive | CD-ROM | Monitor |
| PB75C-AA | Windows NT | Included1 | 120 V | Included2 | Required | Required | Required | Required |
| PB75C-AB | Windows NT | Mandatory | Mandatory | Required | Required | Required | Required | Required |
| PB75C-FA | DIGITAL UNIX | FIS3 | 120 V | Included | Required | Required | Recommended | Recommended |
| PB75C-FB | DIGITAL UNIX | FIS3 | Mandatory | Recommended | Required | Required | Recommended | Recommended |
| PB75C-MA | OpenVMS | FIS3 | 120 V | Included | Required | Required | Recommended | Recommended |
| PB75C-MB | OpenVMS | FIS3 | Mandatory | Recommended | Required | Required | Recommended | Recommended |

1.	Windows NT Server license, media (CD-ROM) kit North American English.

2.	101-key, PC style, North American English, keyboard.

3.		DIGITAL UNIX and OpenVMS Base systems ordered with a minimum of a 1.05 GB disk, include factory installed software (FIS).

KEY:	Mandatory items must be on purchase order at initial order acceptance

Required items are essential for full system operation

Recommended items enhance system functionality

FIS = Factory Installed Software

### Step 1a—AlphaServer 1000 Rackmount Systems

- Windows NT North American variants include Windows NT license, media (CD-ROM) kit, North American English

- Selection of language specific Windows NT license, media (CD-ROM) kit is mandatory for all non-North American variants.

- DIGITAL UNIX and OpenVMS Packaged systems include factory installed software (FIS) on hard disk drive.  Base systems ordered with minimum of 1.05 Gbyte hard disk drive include factory installed software.

- See Uninterruptible Power Supplies (UPS) information following System Specifications.

- Options ordered will be factory installed unless specified as spares.

Note:	AlphaServer 1000 rackmount systems do not support dual power supply configurations

Rackmount Systems include

- Alpha microprocessor 21164 300 MHz CPU with 2 MB ECC on-board cache

- Rackmountable enclosure with

- Ten expansion slots: Seven EISA slots, two PCI slots and one PCI/EISA combination slot

- Integrated Fast Narrow Single Ended SCSI-2 controller with DMA and external SCSI-2 connector.

- Integrated SVGA graphics controller

- 20 industry-standard SIMM slots

- 6 internal storage slots:
One dedicated diskette slot
One CD-ROM slot
One additional removable media slot
Three fixed hard disk drive slots

- 450-Watt self-sensing, self-switching (120/240V, 60/50Hz) power supply

- Two serial ports, support full duplex asynchronous modem control

- One bi-directional enhanced parallel port

- PS/2 style keyboard port and mouse port

- 1.44 Mbyte diskette drive (RX23L) in dedicated slot.

- Three-button mouse.

- 15-foot Power cord (120V/240V as appropriate).

- Rackmount hardware (shelf and rails)

- English Language documentation kit.

- EISA Configuration Utility

- Integrated Server Management.

- Hardware Warranty: Three-year on-site*

- Software Warranty: 90-day SPD conformance with advisory telephone support*

- Windows NT Server plus 5-client access license, media (CD-ROM) kit or†

- DIGITAL UNIX 2-user base license, DIGITAL NAS Base Server 200 license, or

- OpenVMS base license with System Manager license, 
DIGITAL NAS Base Server 200 license.

*	Service upgrades are available; see Step 12, Hardware and Software Supplemental Services on AlphaServer 1000 4/266 deskside system menu.

†	Windows NT language specific media kit is mandatory for non-North American variants; see Step 9.

Rackmount Packaged Systems include

- PCI based Ethernet (DE435-AA)—uses one PCI slot.

- One 2.1 Gbyte narrow hard disk drive

- One memory option, see below

- 600 Mbyte CD-ROM—uses one removable media slot.

- North American variants include a 101-key, PC style North American English keyboard.  Select country-specific keyboard for non-North American variants, see Step 10.

- Cabinet enclosure available separately, see Step 11.

| 5/300 Rackmount Packaged systems |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Hard Drive | Monitor |
| PB75P-AA | Windows NT | Included* | 120 V | Included | 64 MB | 2.1 GB | Required |
| PB75P-AB | Windows NT | Mandatory | 240 V | Required | 64 MB | 2.1 GB | Required |
| PB75P-FA | DIGITAL UNIX | FIS | 120 V | Included | 64 MB | 2.1 GB | Recommended |
| PB75P-FB | DIGITAL UNIX | FIS | 240 V | Recommended | 64 MB | 2.1 GB | Recommended |
| PB75P-MA | OpenVMS | FIS | 120 V | Included | 64 MB | 2.1 GB | Recommended |
| PB75P-MB | OpenVMS | FIS | 240 V | Recommended | 64 MB | 2.1 GB | Recommended |

*	Windows NT Server license, media (CD-ROM) kit North American English.

KEY:	Mandatory items must be on purchase order at initial order acceptance

Required items are essential for full system operation

Recommended items enhance system functionality

FIS = Factory Installed Software

### Step 1a—AlphaServer 1000 Rackmount Systems (continued)

Rackmount Base System

- Base systems include 120V or 240V power cord depending on variant chosen.

- Cabinet enclosure available separately, see Step 11.

| 5/300 Rackmount Base Systems |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order Number | Operating System | O/S Media | Power cord | Keyboard | Memory | Hard Drive | CD-ROM | Monitor |
| PB75S-AA | Windows NT | Included1 | 120 V | Included2 | Required | Required | Required | Required |
| PB75S-AB | Windows NT | Mandatory | 240 V | Required | Required | Required | Required | Required |
| PB75S-FA | DIGITAL UNIX | FIS3 | 120 V | Recommended | Required | Required | Recommended | Recommended |
| PB75S-FB | DIGITAL UNIX | FIS3 | 240 V | Recommended | Required | Required | Recommended | Recommended |
| PB75S-MA | OpenVMS | FIS3 | 120 V | Recommended | Required | Required | Recommended | Recommended |
| PB75S-MB | OpenVMS | FIS3 | 240 V | Recommended | Required | Required | Recommended | Recommended |

1.	Windows NT Server license, media (CD-ROM) kit North American English

2.	101-key, PC style, North American English, keyboard.

3.		DIGITAL UNIX and OpenVMS Base systems ordered with a minimum of a 1.05 GB disk, include factory installed software (FIS).

KEY:	Mandatory items must be on purchase order at initial order acceptance

Required items are essential for full system operation

Recommended items enhance system functionality

FIS = Factory Installed Software

### Step 1b—AlphaServer 1000 Cabinet Systems

5/300 Cabinet Packaged systems include

- Cabinet enclosure (H9A10 120 V or 240 V) with AlphaServer 1000 5/300 Rackmount Packaged System, plus

- One BA36R-AF front-mounted StorageWorks shelf*

- AlphaServer 1000 Rackmount Cabinet Systems ship with dual (2) H7600-xx cabinet power distribution units and the following power cords.

- 120 V systems (H7600-AA) = 2 x 24A terminated with NEMA L5-30P (plugs) and require NEMA L5-30R (receptacles)

- 240 V systems (H7600-AB) = 2 x 16A terminated with NEMA L6-20P (plugs) and require NEMA L6-20R (receptacles)

- H9A10-CE/-CF Cabinet Dimensions

- Outside: 66.9-inches high, 23.62-inches wide, 
33.8-inches deep

- Usable internal rackmount: 56-inches high, 19-inches wide, 32.2-inches deep

*	BA36R-SF StorageWorks wide shelf requires a Fast Wide SCSI controller and SCSI cable, see Step 4b. Select SCSI devices for installation in BA36R-SF StorageWorks shelf from Step 4e.

| 5/300 Cabinet Packaged systems |  |
| --- | --- |
| Order Number | Description |
| PB75R-AA* | Windows NT Rackmount Packaged system (PB75P-AA) |
| PB75R-AB | Windows NT Rackmount Packaged system (PB75P-AB) |
| PB75R-FA/FB | DIGITAL UNIX Rackmount Packaged system (PB75P-Fx) |
| PB75R-MA/MB | OpenVMS Rackmount Packaged system (PB75P-Mx) |

*	Windows NT Server license, media (CD-ROM) kit North American English.

Step 2—Memory

- Packaged Systems include 64 MB memory (one PB7MA-CC) memory kit.

- System supports up to four memory kits (each kit includes industry-standard SIMMs and ECC support).

- System maximum of 1 GB can be obtained by selecting Base System and four PB7MA-CE 256 MB memory kits.

| PB7MA-CB | 32 MB 70ns (8 MB SIMMs) memory kit |
| --- | --- |
| PB7MA-CC | 64 MB 70ns (16 MB SIMMs) memory kit |
| PB7MA-CD | 128 MB 70ns (32 MB SIMMs) memory kit |
| PB7MA-CE | 256 MB 70ns ( 64 MB SIMMs) memory kit |

| DJ-ML200-BA | PCI-based 4 MB PrestoServe I/O performance enhancement option for DIGITAL UNIX system only, maximum one per system |
| --- | --- |

Step 3—Monitors

- Windows NT systems require a graphics monitor to run all functions.

- EISA Configuration Utility (ECU) is accessible via the console port for DIGITAL UNIX and OpenVMS systems. Optional graphics capability is available. Select graphics adapter, monitor, and country specific keyboard for DIGITAL UNIX and OpenVMS systems if required.

- Graphics monitors other than those listed below can be used if compatible with graphics adapter included with system.

- OpenVMS supports 640 x 480 x 256 @ 60Hz, 800 x 600 x 256 @ 60Hz

- DIGITAL UNIX supports 640 x 480 x 256 @ 60 Hz, 800 x 600 x 256 @ 60Hz

- Windows NT supports 640 x 480 x 256 @ 60 Hz, 640 x 480 x 256 @ 72 Hz, 800 x 600 x 256 @ 56 Hz, 800 x 600 x 256 @ 60 Hz, 800 x 600 x 256 @ 72 Hz.

Note: Higher resolution available with optional EISA or PCI graphics adapters (see Step 5).

| VRC15-KA/K4 | 15” (13.9” viewable image size) high-resolution color monitor with Light Gray enclosure. Flat-square CRT with 0.28 mm dot pitch and anti-reflection, anti-glare, anti static coating. Auto-scanning from VGA to 1024 x 768 at 75Hz NI refresh rates. MPR-II, Energy Star, DPMS and NUTEK compliant. 120/240V universal power supply. Includes 1.4 meter HD15 male to male video cable. Select -KA for Northern Hemisphere or -K4 for Southern Hemisphere operation. If purchased in North America, -KA includes 120V power cord, otherwise power cords for -KA and -K4 must be ordered separately, see Step 10. |
| --- | --- |

| VRT17-PA/P4 | 17” (16.0” viewable image size) high-resolution color monitor with Light Gray enclosure. Trinitron aperture grille CRT with 0.26 mm stripe pitch and anti-reflection, anti-glare, anti static coating. Auto-scanning from VGA to 1280 x 1024 at 75Hz NI refresh rates. MPR-II, Energy Star, DPMS and NUTEK compliant. 120/240V universal power supply. Includes 3.0 meter HD15 male to BNC video cable. Select -PA for Northern Hemisphere or -P4 for Southern Hemisphere operation. If purchased in North American, -PA includes 120V power cord, otherwise power cords for -PA and -P4 must be ordered separately, see Step 10. |
| --- | --- |

| VRC21-LA/W4 | 21” (19.6” viewable image size) ultra high-resolution color monitor with Light Gray enclosure. Diamondtron aperture grille CRT with 0.30 mm stripe pitch and anti-reflection, anti-glare, anti static coating. Auto-scanning from VGA to 1600 x 1200 at 75Hz NI refresh rates. On Screen display (OSD). Stereo viewing compatible. MPR-II, Energy Star, DPMS and NUTEK compliant. 120/240V universal power supply. Includes 3.0 meter HD15 male to BNC video cable. Select -LA for Northern Hemisphere, or -W4 for Southern Hemisphere operation. If purchased in North America, -LA includes 120V power cord, otherwise power cords for -LA and -W4 must be ordered separately, see Step 10. |
| --- | --- |

Step 4—Storage

Configuration rules for Pedestal Internal Storage

- Integral Fast Narrow Single Ended (FNSE) SCSI-2 controller supports maximum of seven devices:

- Two internal 5.25-inch removable media devices

- Four 3.5-inch hard disk drives in internal storage assembly

- One external storage device.

- Pedestal enclosure supports seven 3.5-inch narrow disk drives in internal storage assembly

- Internal storage assembly is normally configured for split-bus with four drives on first bus and three drives on second bus. By attaching the jumper cable and moving the terminator internal storage assembly can be re-configured for single-bus mode with support for a maximum of seven disk drives on one additional SCSI adapter.

- An additional SCSI-2 controller is required to support internal storage assembly if set for split-bus mode.

Configuration rules for Rackmount Internal Storage

- Integral Fast Narrow Single Ended (FNSE) SCSI-2 controller supports maximum of seven devices:

- Two internal 5.25-inch removable media devices

- Three internal 3.5-inch disk drives

- Two external storage devices.  Maximum external cable length cannot exceed 1.5 meters.

- Rackmount enclosure supports three 3.5-inch narrow disk drives.

Step 4a—Internal Storage

All Pedestal and Rackmount systems include 1.44 MB diskette drive in dedicated slot.

- Pedestal and Rackmount Packaged systems include:

- One CD-ROM drive

- One 2.1 GB narrow hard drive

Removable Media Devices

| PBXRD-DA | 600 MB 5.25 inch half-height 12X CD-ROM drive (RRD46) |
| --- | --- |
| PBXTZ-AA | 2.0 GB 5.25-inch half-height SCSI QIC tape drive (TZK11) |
| PBXTL-DA | 8.0 GB 5.25-inch half-height SCSI 4 mm DAT drive (TLZ09) |

Pedestal System Hard Drives

Wide disk drives require wide SCSI-2 controllers to operate in 16-bit wide mode.

| RZ26N-VA | 1.05 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive |
| --- | --- |
| RZ28D-VA | 2.1 GB 8-bit narrow 7200 RPM 3.5 x 1” SCSI hard disk drive |
| RZ28M-VA | 2.1 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive |
| RZ29B-VA | 4.3 GB 8-bit narrow 7200 RPM 3.5 x 1.6” SCSI hard disk drive |

| RZ26N-VW | 1.05 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI hard disk drive |
| --- | --- |
| RZ28D-VW | 2.1 GB 16-bit wide 7200 RPM 3.5 x 1” SCSI hard disk drive |
| RZ28M-VW | 2.1 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI hard disk drive |
| RZ29B-VW | 4.3 GB 16-bit wide 7200 RPM 3.5 x 1.6” SCSI hard disk drive |

Pedestal system internal SCSI cables and terminators

| PB7HA-AA | Internal SCSI cable and terminator for FNSE SCSI controllers and 8-bit narrow devices (50 pin) |
| --- | --- |
| PB7HA-BB | Internal SCSI cable and terminator for FNSE SCSI controllers and 16-bit wide devices (68-pin) |
| PB7HA-BA | Internal SCSI cable and terminator for FWSE SCSI controllers and 16-bit wide devices (68-pin) |

Rackmount System Hard Disk Drives (internal only)

| PBXRZ-EB | 1.05 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive (RZ26N) |
| --- | --- |
| PBXRZ-JB | 2.1 GB 8-bit narrow 7200 RPM 3.5 x 1” SCSI disk drive (RZ28D) |
| PBXRZ-NA | 4.3 GB 8-bit narrow 7200 RPM 3.5 x 1.6” SCSI hard disk drive (RZ29B) |

Step 4b—Storage Controllers

- Maximum two KZESC-xx one- and three-port EISA-based RAID controllers supported per system.

- KZESC-xx StorageWorks RAID Array 210 includes EISA backplane RAID controller and StorageWorks RAID Array 210 Subsystem family software and documentation kit for OpenVMS, DIGITAL UNIX and Windows NT.

- KZESC-xx controller installed in Pedestal system requires a PB7HA-AA SCSI cable kit for connection to internal StorageWorks shelf, two kits required for split bus mode.

- Maximum number of EISA-based controllers of all types is limited by the total number of available EISA slots and available IRQs.

- Maximum of three KZPSC-xx one- and three port PCI-based RAID controllers supported per system.  Note: Packaged systems have two PCI slots available due to installed PCI-based Ethernet controller.

- KZPSC-xx controllers are supported in PCI slots 0, 1 and 2 only

- KZPSC-xx StorageWorks RAID Array 230 includes PCI backplane RAID controller and StorageWorks RAID Array 230 Subsystem family software and documentation kit for OpenVMS, DIGITAL UNIX and Windows NT.

- KZPSC-AA controller installed in Pedestal system requires a PB7HA-BA SCSI cable kit for connection to internal StorageWorks shelf, two kits required for split bus mode.

- KZPSA-BB PCI-based one-port Fast Wide Differential SCSI controller supports externally connected wide disks in StorageWorks enclosures and Rackmount shelves with DWZZB signal converter, or narrow disks in narrow StorageWorks enclosures and Rackmount shelves with DWZZA signal converter. Internal storage assembly hard drives are not supported on Fast Wide Differential Controller.

- Each controller requires one bus slot, except KFESB which requires two bulkhead slots when system is middle node in an OpenVMS cluster.

- SCSI cables are not included and must be ordered separately.

- External DSSI cables are not included and must be ordered separately.

SCSI Controllers

Note:	Pedestal systems require SCSI terminator and cable kit to connect SCSI controller to internal storage assembly:

- PB7HA-AA from FNSE SCSI-2 controller to 8-bit narrow devices in internal storage assembly(50 pin)

- PB7HA-BA from FWSE SCSI-2 controller to 16-bit wide devices in internal storage assembly(68-pin)

- PB7HA-BB from FNSE SCSI-2 controller to 16-bit wide devices in internal storage assembly (68-pin)

Rackmount Cabinet systems include KZPAA FNSE SCSI-2 controller and one BN21H-02 SCSI cable to connect to BA35R-SF StorageWorks shelf.

| KZPAA-AA | PCI-based one-port high-performance Fast Narrow Single Ended (FNSE) SCSI-2 controller |
| --- | --- |
| BN21H-xx | Connects from KZPAA-AA to narrow StorageWorks enclosures and narrow StorageWorks rackmount shelves. Pedestal system requires PB7HA-AA SCSI cable kit for connection to internal StorageWorks shelf |

| KZPDA-AA | PCI-based one-port Fast Wide Single Ended (FWSE) SCSI controller |
| --- | --- |
| BN21K-02 | Connects from KZPDA to wide StorageWorks enclosures and wide StorageWorks rackmount shelves. |

| KZPSA-BB | PCI-based one-port high-performance Fast Wide Differential (FWD) SCSI controller. Supports external disks only. |
| --- | --- |
| BN21K-01 | Connects from KZPSA to DWZZA-VA in narrow StorageWorks enclosures and narrow StorageWorks rackmount shelves, or DWZZB-VW in wide StorageWorks enclosures and wide StorageWorks rackmount shelves |

| KZESC-AA* | One-port EISA backplane RAID (FNSE) controller; includes StorageWorks RAID Array 210 Subsystem family software and documentation kit. Pedestal system requires PB7HA-AA SCSI cable kit for connection to internal StorageWorks shelf |
| --- | --- |
| BN21H-02 | Connects from KZPAA-AA to narrow StorageWorks enclosures and narrow StorageWorks rackmount shelves |
| BN21N-02 | Connects from KZESC-BA to wide StorageWorks enclosures and wide StorageWorks rackmount shelves |

Step 4b—Storage Controllers (continued)

| KZESC-BA* | Three-port EISA backplane RAID controller; includes StorageWorks RAID Array 210 Subsystem family software and documentation kit. Pedestal system requires PB7HA-AA SCSI cable kit for connection to internal StorageWorks shelf. |
| --- | --- |
| CK-SWXCR-AA | SCSI cable/bulkhead assembly kit for KZESC-BA, required for connecting second and third ports to builkhead slot |
| BN21H-02 | Connects from KZESC-BA to narrow StorageWorks enclosures and wide StorageWorks rackmount shelves |
| BN21N-02 | Connects from KZESC-BA to wide StorageWorks enclosures and wide StorageWorks rackmount shelves |

| KZPSC-AA* | One-port PCI backplane RAID Fast Wide Single Ended (FWSE) controller; includes StorageWorks RAID Array 230 Subsystem family software and documentation kit. Pedestal system requires PB7HA-BA SCSI cable kit for connection to internal StorageWorks shelf. |
| --- | --- |
| BN31S-1E | Connects from KZPSC-AA to wide StorageWorks enclosures and wide StorageWorks rackmount shelves |
| BN31L-1E | Connects from KZPSC-AA to narrow StorageWorks enclosures and narrow StorageWorks rackmount shelves |

| KZPSC-BA* | Three-port PCI backplane RAID (FWSE) controller; includes StorageWorks RAID Array 230 Subsystem family software and documentation kit. Pedestal system requires PB7HA-BA SCSI cable kit for connection to internal StorageWorks shelf. |
| --- | --- |
| BN31K-0E | SCSI cable/bulkhead assembly kit for KZPSC-BA; required for connection to third port; uses one PCI bulkhead slot. |
| BN31S-1E | Connects from KZPSC-BA to wide StorageWorks enclosures and wide StorageWorks rackmount shelves |
| BN31L-1E | Connects from KZPSC-BA to narrow StorageWorks enclosures and narrow StorageWorks rackmount shelves |

| KZPSM-AA | PCI-based SCSI Ethernet combination controller. (FWSE) SCSI controller, 10 Mbit Ethernet. |
| --- | --- |
| BC25V-1A | SCSI cable/bulkhead assembly kit for KZPSM-AA; connects from KZPSM-AA to bulkhead for external storage; requires BN21K-xx SCSI cable |
| BN21K-xx | Connects from KZPSM-AA bulkhead assembly kit wide StorageWorks enclosures and wide StorageWorks rackmount shelves |

*	See Storage Devices for additional information on StorageWorks RAID Array 2x0 Subsystems

DSSI Controllers

| KFPSA-AA | PCI-based single-DSSI controller (OpenVMS systems only). See Step 4f DSSI cables. |
| --- | --- |

| KFESB-AA | EISA-based single-DSSI controller (OpenVMS systems only). Uses one EISA slot if system is end node in OpenVMS cluster, or two slots if system is middle node in OpenVMS cluster. Maximum two (if end node in cluster) or maximum of one (if middle node in cluster). See Step 4f DSSI cables. |
| --- | --- |

Step 4c—External Tape Expansion

- Integral Fast Narrow SCSI-2 controller can be extended outside the system enclosure via the SCSI-out port to support external SCSI devices. Maximum external bus length including cable and device cannot exceed 1.0 meter.

- External tape drives are also supported on optional PCI-based high-performance SCSI controllers; KZPAA-AA (maximum external bus length including cable and tape device cannot exceed 3.0 meter) and KZPSA-BB (maximum external bus length including cable and tape device cannot exceed 25.0 meters).

- External tape drives are not supported on one- and three-port (KZESC-xx) high performance Fast-SCSI-2 controllers.

- External tape drives supported on one- and three-port (KZPSC-xx) high performance SCSI controllers on Windows NT only.

- Each tabletop tape device requires a three-foot high density 50-pin to low density 50-pin SCSI cable (BN23G-0E).

External Tapes supported on Windows NT servers

| TLZ09 | 8.0 GB 4 mm DAT tape drive |
| --- | --- |
| TLZ7L1,2 | 32.0 GB 4 mm DAT autoloader |
| TZK11-DA | 2.0 GB 5.25-inch tabletop QIC tape drive |
| TZ872 | 20 GB, DLT tape drive |
| TZ8752 | 100 GB, DLT tape autoloader |
| TZ8772 | 140 GB, DLT tape autoloader |
| BN23G-0E | 3 Foot Molded SCSI Cable, required for each tabletop tape device |

External Tapes supported on DIGITAL UNIX and OpenVMS servers

| TLZ093 | 8.0 GB 4 mm DAT tape drive |
| --- | --- |
| TLZ7L1,2,3 | 32.0 GB 4 mm DAT autoloader |
| TZK11-DA3 | 2.0 GB 5.25-inch tabletop QIC tape drive |
| TKZ60-FA/FC | 400 MB IBM 3480/3490 compatible tabletop tape drive |
| TKZ60-EA3 | 400 MB IBM 3480/3490 compatible tabletop tape drive |
| TKZ61 | 400 MB IBM compatible tape autoloader |
| TKZ61-AC3 | 400 MB IBM compatible tape autoloader |
| TKZ62 | 2.4 GB IBM compatible tape autoloader |
| TKZ62-AC3 | 2.4 GB IBM compatible tape autoloader |
| TKZ15-TA | 10 GB, 8 mm, tabletop tape drive |
| TKZ15-VA3,5 | 10 GB, 8 mm, tabletop tape drive |
| TSZ07-CA | 40/140 MB, reel/reel, tabletop tape drive |
| TSZ07-AA3 | 40/140 MB, reel/reel, tabletop tape drive |
| TZ872 | 20 GB, DLT tape drive |
| TZ87-VA3 | 20 GB, DLT tape drive |
| TZ8752 | 100 GB, DLT tape autoloader |
| TZ8772 | 140 GB, DLT tape autoloader |
| TZ882 | 20/40 GB, DLT tape drive |
| TKZ9E-TA4 | 2/5/7/10/14 GB 8 mm helical scan tape drive, tabletop |
| TKZ9E-VA3 | 2/5/7/10/14 GB 8 mm helical scan tape drive in StorageWorks SBB carrier |
| BN23G-0E | 3 Foot Molded SCSI Cable, required for each tabletop tape device. |

1	Includes four cartridge loader. Larger magazines are supported.

2	Base operating systems support sequential back-up mode only; additional software is required for random access backup. See Storage Devices for details.

3	Variants for use in Rackmount systems

4	8 mm drive requires OpenVMS MK-driver patch when connected to KZPSA controller in OpenVMS system.

5	Windows NT require layered driver software QB-4STAA-SA

Step 4d—External Disk Expansion for Pedestal Systems

- BA353 expansion units are supported on wide and narrow SCSI controllers.  Devices operate in narrow mode when BA353 is connected to a Fast Wide SCSI controller.

- BA356 and BA346 expansion units are supported on Fast Wide SCSI controllers.

StorageWorks Modular Storage Options

| BA353-AA | StorageWorks 8-bit Narrow Desktop expansion unit includes enclosure and 120 V power cord. Supports up to three 3.5” narrow hard disk drives. Not supported with RAID controllers. |
| --- | --- |
| BA356-KC | StorageWorks 16-bit Wide Pedestal expansion unit includes BA356 basic shelf, BA35X-HA universal ac power supply, pedestal mounting kit, and 120 V power cord. Supports up to seven 3.5” wide or narrow drives; narrow drives operate in narrow mode. |
| BA346-KB | StorageWorks 16-bit Wide Pedestal expansion unit includes BA356 basic shelf, BA35X-HA universal ac power supply, pedestal mounting kit, and 120 V power cord. Supports up to nine devices, two 5.25” narrow and seven 3.5” wide or narrow drives; narrow drives operate in narrow mode. |
| DWZZA-VA | Bi-directional SCSI signal converter and bus extender kit. 16 bit Fast Wide Differential SCSI-2 on one end and 8 bit Fast Narrow Single-Ended SCSI-2 on other end. |
| DWZZB-VW | Bi-directional SCSI signal converter and bus extender kit. 16 bit Fast Wide Differential SCSI-2 on one end and 8 or 16 bit Fast Wide or Fast Narrow Single-Ended SCSI-2 on other end. |

See Storage Device for additional ordering information for StorageWorks modular storage expansion and supported devices.

Step 4e—External Disk Expansion for Rackmount and Cabinet Systems

- External BA35R Rackmount StorageWorks shelves are supported on all Fast SCSI-2 controllers.

- External BA36R Rackmount StorageWorks shelves are supported on Fast Wide SCSI controller.

- AlphaServer 1000 Rackmount systems provide four SCSI expansion ports on rear panel of system.

- Order BN21H-02 cable to connect a single BA35R StorageWorks shelf to controllers.

- Order BN31*-xx cable to connect a single BA36R StorageWorks shelf to controllers.

Rackmount StorageWorks Shelves

StorageWorks BA35R/BA36R 8-bit and 16-bit Rackmount Shelves

| BA36R-AF | Front mount BA356 Rackmount StorageWorks Shelf, BA35R-MH 16-bit I/O module, BA35X-HF power supply |
| --- | --- |
| BA36R-AR | Rear mount BA356 Rackmount StorageWorks Shelf, BA35R-MH 16-bit I/O module, BA35X-HF power supply |
| BA35R-SF | Front Mount BA356 Rackmount StorageWorks Shelf, BA35X-MG 8-bit I/O module, BA35X-HF power supply |
| BA35R-SR | Rear Mount BA356 Rackmount StorageWorks Shelf, BA35X-MG 8-bit I/O module, BA35X-HF power supply |

| DWZZA-VA | Bi-directional SCSI signal converter and bus extender kit. 16 bit Fast Wide Differential SCSI-2 on one end and 8 bit Fast Narrow Single-Ended SCSI-2 on other end. Requires BN21K-** cable. |
| --- | --- |
| DWZZB-VW | Bi-directional SCSI signal converter and bus extender kit. 16 bit Fast Wide Differential SCSI-2 on one end and 8 or 16 bit Fast Wide Single-Ended SCSI-2 on other end. Requires BN21K-** cable. |

Hard Disk Drives for Rackmount StorageWorks Shelves (External only)

- Wide drives require wide controller to operate in 16-bit wide mode.

- Wide drives are supported in BA36R Rackmount StorageWorks shelves

- Narrow drives are supported in BA35R Rackmount StorageWorks shelves

| RZ26N-VA | 1.05 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive |
| --- | --- |
| RZ28M-VA | 2.1 GB 8-bit narrow 5400 RPM 3.5 x 1” SCSI hard disk drive |
| RZ28D-VA | 2.1 GB 8-bit narrow 7200 RPM 3.5 x 1” SCSI hard disk drive |
| RZ29B-VA | 4.3 GB 8-bit narrow 7200 RPM 3.5 x 1.6” SCSI hard disk drive |

Step 4e—External Disk Expansion for Rackmount and Cabinet Systems (continued)

Hard Disk Drives for Rackmount StorageWorks Shelves (External only)

| RZ26N-VW | 1.05 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI hard disk drive |
| --- | --- |
| RZ28M-VW | 2.1 GB 16-bit wide 5400 RPM 3.5 x 1” SCSI hard disk drive |
| RZ28D-VW | 2.1 GB 16-bit wide 7200 RPM 3.5 x 1” SCSI hard disk drive |
| RZ29B-VW | 4.3 GB 16-bit wide 7200 RPM 3.5 x 1.6” SCSI hard disk drive |

*	Wide drives operate in 8-bit narrow mode when connected to integral Fast Narrow Single Ended SCSI-2 controller

Step 4f—DSSI Cables

- EISA-based DSSI controller (KFESB-AA) uses “Micro-Ribbon” connection.

- KFESB to any external “Pin-Socket” DSSI connection (VAX 4000s, R400X) requires BC22Q-xx DSSI cable.

- KFESB to any external “Micro-Ribbon” DSSI straight connection (all other DSSI systems and storage devices requiring straight connection) requires BC21Q-xx DSSI cable.

- KFESB to any external “Micro-Ribbon” DSSI right-angle connection (all other DSSI systems and storage devices requiring right-angle connection) requires BC29S-xx DSSI cable.

Step 5—Graphics Adapters

- Integrated SVGA graphics controller supports the following monitors:

- DIGITAL UNIX supports 640 x 480 x 256 @ 60 Hz monitors, 800 x 600 x 256 @ 60 Hz monitors

- OpenVMS supports 640 x 480 x 256 @ 60 Hz monitors, 800 x 600 x 256 @ 60 Hz monitors

- Windows NT supports 640 x 480 x 256 @ 60 Hz, 640 x 480 x 256 @ 72 Hz, 800 x 600 x 256 @ 56 Hz, 800 x 600 x 256 @ 60 Hz, 800 x 600 x 256 @ 72 Hz

- EISA Configuration Utility (ECU) is accessible via the console port for DIGITAL UNIX and OpenVMS systems. Optional graphics capability is available. Select graphics adapter, monitor, and country specific keyboard for DIGITAL UNIX and OpenVMS systems if required.

| PB2GA-JB | PCI based Trio64 S3 1 MB DRAM graphics adapter 1024 x 768 (OpenVMS, Windows NT) |
| --- | --- |
| PB2GA-JA | PCI based Trio64 S3 2-MB DRAM graphics adapter 1024 x 768 (DIGITAL UNIX, OpenVMS, Windows NT) |
| PB2GA-FB | ISA based ATI Mach64 2 MB DRAM graphics adapter 1280 x 1024 (DIGITAL UNIX, OpenVMS, Windows NT) |
| PBXGA-AA | PCI-based ZPXp-E1 8-plane 1024 x 768 graphics adapter (DIGITAL UNIX and OpenVMS) |
| PBXGA-AN | PCI-based ZPXp-E1 8-plane 1024 x 768 graphics adapter (Windows NT) |
| PBXGA-BA | PCI-based ZPXp-E2 24-plane 1024 x 768 graphics adapter (DIGITAL UNIX and OpenVMS) |
| PBXGA-BN | PCI-based ZPXp-E2 24-plane 1024 x 768 graphics adapter (Windows NT) |

Step 6—Networks and Communications

- Packaged systems include PCI-based Ethernet controller, uses one PCI slot

- Two additional PCI-based controllers supported per system.

- Select networking cable for Ethernet controller

- BNE4G-02 for AUI

- BN26K-xx for 10BaseT (twisted pair)

- BC16M-xx for ThinWire

- Maximum number of each EISA-based network controllers supported per system:

- Three DE425-AA, Three DEFEA-xA

- Four DW300-AA, Four DNSES

| DE435-AA | PCI-based DIGITAL Etherworks 32-bit high-performance network interface card |
| --- | --- |
| DE500-XA | PCI-based Fast Ethernet network interface card (see PCI Option Slot Table) |
| DEFPA-AB* | PCI-based DEC FDDIcontroller, Single Attachment |
| DEFPA-DB* | PCI-based DEC FDDIcontroller, Dual Attachments |
| DEFPA-UB* | PCI-based DEC FDDIcontroller, Single Attachment (UTP) |
| DGLPB-AB | PCI-based ATMworks 350 adapter |
| PBXNP-AA | PCI-based Token Ring adapter, no boot support. DIGITAL UNIX and OpenVMS systems require driver floppy. Not supported on Windows NT. |

| DE425-AA | EISA-based Ethernet, OpenVMS and DIGITAL UNIX only |
| --- | --- |
| DEFEA-AA | EISA-based DEC FDDIcontroller, Single Attachment |
| DEFEA-UA | EISA-based DEC FDDI (UTP) controller |
| DEFEA-DA | EISA-based DEC FDDIcontroller, Dual Attachment (requires two slots) |
| DW300-AA | EISA-based Token-Ring adapter includes NetWare V2.15 driver, LAN Manager Driver, and documentation (Not supported by DECnet/OSI for OpenVMS) |
| DNSES-AA | EISA-based synchronous communications controller, DIGITAL UNIX and OpenVMS systems only |

| CXI01-AA | ISA Asynchronous MUX Adapter, 16 lines. Expandable to 64 lines. Supported on Windows NT and DIGITAL UNIX only. |
| --- | --- |
| CXI01-AD | ISA Asynchronous MUX Adapter, 16 lines. Expandable to 224 lines. Supported on Windows NT and DIGITAL UNIX only. |
| PBXDI-AA | ISA-based Two Port Synchronous Communications controller with interface support for EIA-232/V.24/V.28, Windows NT only |
| PBXDI-AB | ISA-based Two Port Synchronous Communications controller with interface support for V.35, Windows NT only |
| PBXDI-AC | ISA-based Two Port Synchronous Communications controller with interface support for X.21 and EIA-530, Windows NT only |
| DIIAA-AA | ISA-based ISDN terminal adapter (U.S.); Windows NT only |
| DIIAA-AB | ISA-based ISDN terminal adapter (non-U.S.); Windows NT only |

*	Supported as data device only.

Step 6a—Integrated Telecommunications AlphaServer Option

- Integrated telecommunications options are supported on AlphaServer 1000 Windows NT Pedestal systems only

- Any combination of digital and analog trunk options can be installed in same system in available EISA slots

- Dual Power Supply is not supported when Integrated Telecommunications AlphaServer option is installed; the 48/96V Ring Generating Power supply occupies second power supply slot.

- See separate menu for Integrated Telecommunications Options for AlphaServer 1000 Pedestal systems for additional configuration information.

| PB71T-AB | 12 64 Kbps analog trunks by 24 client telephone stations, includes (1) ISA 12 port analog (Loop Start) trunk card (uses one EISA slot) (1) ISA 24 port POTS station card (uses one EISA slot) Mitel MediaPath Windows NT license and media, Integrated Telecommunications Services Windows NT license, documentation and media, DECtalk runtime license, MVIP cable -48/96V Ring Generating Power Supply (PBXCS-AA) and chassis assembly, 125 V North American power cord |
| --- | --- |
| PB7IT-AC1 | 24 64 Kbps digital trunks by 24 client telephone stations, includes (1) ISA T1 dual DSP trunk card (uses 1 EISA slot and blocks one PCI slot)1 (1) ISA 24 port POTS station card (uses one EISA slot) Mitel MediaPath Windows NT license and media, Integrated Telecommunications Services Windows NT license, media and documentation, DECtalk runtime license, MVIP cable -48/96V Ring Generating Power Supply (PBXCS-AA) and chassis assembly, 125 V North American power cord |
| PB7IT-AD1 | 48 64 Kbps digital trunks by 96 client telephone stations, includes (1) ISA T1 quad DSP trunk card (uses 1 EISA slot and blocks one PCI slot)1 (4) ISA 24 port POTS station cards (uses 4 EISA slots) Mitel MediaPath Windows NT license and media, Integrated Telecommunications Services Windows NT license, media and documentation, DECtalk runtime license, MVIP cable -48/96V Ring Generating Power Supply (PBXCS-AA) and chassis assembly, 125 V North America power cord |

1	T1 card should be installed in first EISA slot adjacent to PCI slots, otherwise it consumes two EISA slots. Installation in first EISA slot blocks one adjacent PCI bulkhead slot, this reduces the number of available PCI slots by one.

Step 7—Additional Power Supply—Pedestal Systems Only

Note:	AlphaServer 1000 Rackmount and Cabinet systems do not support dual power supply configurations

- Additional power supply may be added to AlphaServer 1000 Pedestal system for n+1 redundancy

- Note: AlphaServer 1000 Windows NT Pedestal systems with Integrated Telecommunications options installed do not support dual power supply configurations; the -48/96V Ring Generating Power supply occupies second power supply slot.

- Country specific power cord must be ordered separately, see Step 10.

| H7290-AA | 450-Watt Redundant Power Supply Option |
| --- | --- |

See UPS information following System Specifications.

Step 8—Terminals and Printers

Systems include two EIA-232 asynchronous serial ports with 9-pin D-subminiature connectors.

Select terminals and serial printers as required. A 9-pin to MMJ adapter (H8571-J) is required for each connection. A cable must be ordered unless otherwise provided.

Step 9—Software

- North American variants of Windows NT Packaged and Base systems include Windows NT Server plus 5-client access license, media (CD-ROM) kit North American English.  Selection of language specific Windows NT media kit is mandatory for non-North American variants.

Windows NT Server plus 5-client access license, media (CD-ROM) kits

| QB-23CAA-SB | Windows NT Server license, media kit American/North American English |
| --- | --- |
| QB-23C8A-SB | Windows NT Server license, media kit International English |
| QB-23CPA-SB | Windows NT Server license, media kit French |
| QB-23CGA-SB | Windows NT Server license, media kit German |
| QB-23CSA-SB | Windows NT Server license, media kit Spanish |
| QB-23CUA-SB | Windows NT Server license, media kit Italian |
| QB-23CJA-SB | Windows NT Server license, media kit Japanese |
| QB-23CDA-SB | Windows NT Server license, media kit Danish |
| QB-23CMA-SB | Windows NT Server license, media kit Swedish |
| QB-23CNA-SB | Windows NT Server license, media kit Norwegian |
| QB-23CFA-SB | Windows NT Server license, media kit Finnish |
| QB-23CHA-SB | Windows NT Server license, media kit Dutch |
| QB-23CVA-SB | Windows NT Server license, media kit Portuguese |
| QB-23C4A-SB | Windows NT Server license, media kit Korean |
| QB-23C3A-SB | Windows NT Server license, media kit Taiwanese |
| QB-23C2A-SB | Windows NT Server license, media kit PRC Chinese |

Windows NT Server Optional Software

| QB-4G45A-AA | Purveyor Web Server Software V1.1 for Process Software Corp. |
| --- | --- |

DIGITAL UNIX Concurrent Use Licenses

Software Processor Code = E

- DIGITAL UNIX Packaged and Base systems require operating system media and documentation for first system on site.

DIGITAL UNIX Concurrent Use Licenses are not specific to a single system and can be moved from one system to another at user discretion.

| QL-MT7AM-3B | DIGITAL UNIX Concurrent Use 1-user license |
| --- | --- |
| QL-MT7AM-3C | DIGITAL UNIX Concurrent Use 2-user license |
| QL-MT7AM-3D | DIGITAL UNIX Concurrent Use 4-user license |
| QL-MT7AM-3E | DIGITAL UNIX Concurrent Use 8-user license |
| QL-MT7AM-3F | DIGITAL UNIX Concurrent Use 16-user license |
| QL-MT7AE-AA | DIGITAL UNIX Traditional unlimited user license |
| QL-MT5AE-AA | DIGITAL UNIX developer's extension license |

DIGITAL UNIX Media and Documentation—required for first system on site

| QA-MT4AA-H8 | DIGITAL UNIX media and on-line documentation on CD-ROM |
| --- | --- |
| QA-MT4AA-GZ | DIGITAL UNIX full hardcopy documentation |

DIGITAL UNIX Layered Products CD-ROM

| QA-054AA-H8 | Layered products media and documentation for DIGITAL UNIX on CD-ROM |
| --- | --- |

Step 9—Software (continued)

DECnet for DIGITAL UNIX

| QL-MTJAE-AA | DECnet/OSI end-system license for DIGITAL UNIX |
| --- | --- |
| QL-MTKAE-AA | DECnet/OSI extended function license for DIGITAL UNIX |

Internet AlphaServer Software for DIGITAL UNIX

- Internet AlphaServer Software consists of license, CD-ROM media, and documentation for software listed below.

- Please reference the Internet AlphaServer 1000 4/233 menu for complete list of available Internet software and services.

| QB-4GQAA-KA | Internet AlphaServer Software (USA and Canada only) |
| --- | --- |
| QB-4GQAA-KB | Internet AlphaServer Software (International) |

OpenVMS Concurrent Use Licenses

Software Processor Code = E

- OpenVMS Packaged and Base systems require operating system media and documentation for first system on site.

OpenVMS Concurrent Use Licenses are not specific to a single system and can be moved between systems at user discretion. OpenVMS Concurrent Use Licenses can also be shared in a mixed OpenVMS VAX and OpenVMS Alpha Cluster.

| QL-MT3AA-3B | OpenVMS Concurrent Use 1-user license |
| --- | --- |
| QL-MT3AA-3C | OpenVMS Concurrent Use 2-user license |
| QL-MT3AA-3D | OpenVMS Concurrent Use 4-user license |
| QL-MT3AA-3E | OpenVMS Concurrent Use 8-user license |
| QL-MT3AA-3F | OpenVMS Concurrent Use 16-user license |
| QL-MT3AA-3G | OpenVMS Concurrent Use 32-user license |
| QL-MT3AA-3H | OpenVMS Concurrent Use 64-user license |
| QL-MT3AA-3J | OpenVMS Concurrent Use 128-user license |
| QL-MT3AA-3K | OpenVMS Concurrent Use 256-user license |
| QL-MT2AE-AA | OpenVMS Traditional unlimited user license |

OpenVMS Media and Documentation—required for first system on site

| QA-MT1AA-H8 | OpenVMS media and on-line documentation CD-ROM |
| --- | --- |
| QA-001AA-GZ | OpenVMS hardcopy documentation |

OpenVMS Layered Products CD-ROM

| QA-03XAA-H8 | Layered products media and documentation for OpenVMS on CD-ROM |
| --- | --- |

DECnet for OpenVMS

| QL-MTGAE-AA | DECnet extended function license for OpenVMS |
| --- | --- |
| QL-MTHAE-AA | DECnet end-system to extended function upgrade license for OpenVMS |

DSSI Information

| EK-410AB-MG | DSSI VMScluster Installation Guide |
| --- | --- |
| EK-D4AXP-TS | DSSI VMScluster Troubleshooting Guide |

Step 10—Power Cords, Keyboards, and Documentation

Pedestal and Monitor Power Cords

- North American variants include BN26J-1K 120 V (North American, Japan) power cord.  If an other power cord is selected, both power cords ship with system.

| BN26J-1K | North American, Japan, 120 V |
| --- | --- |
| BN19H-2E | Australia, New Zealand, 2.5 meters long |
| BN19C-2E | Central Europe, 2.5 meters long |
| BN19A-2E | U.K., Ireland, 2.5 meters long |
| BN19E-2E | Switzerland, 2.5 meters long |
| BN19K-2E | Denmark, 2.5 meters long |
| BN19Z-2E | Italy, 2.5 meters long |
| BN19S-2E | Egypt, India, South Africa, 2.5 meters long |
| BN18L-2E | Israel, 2.5 meters long |

Rackmount Power Cords

- 120V and 240V 15 foot power cords are included with all systems, variant designates power cord voltage (See Step 1).

Note:	AlphaServer 1000 Pedestal system power cords are less then 15-feet long and are not supported in Rackmount cabinet enclosure

Keyboards

- North American variants include LK471-AA keyboard (see Step 1).  For these variants, if an additional keyboard is ordered, both keyboards ship with system.

- LK471-xx are 101 key PC style keyboards.  LK461-xx are 108 key VT style keyboards.

| LK471-A2 | LK461-A2* | North American, Japan (English) |
| --- | --- | --- |
| LK471-AB | LK461-AB | Belgium (French |
| LK471-AD | LK461-AD | Denmark |
| LK471-AE | LK461-AE | United Kingdom (English) |
| LK471-AG | LK461-AG | Germany |
| LK471-AI | LK461-AI | Italy |
| LK471-AK | LK461-AK | Switzerland (Generic) |
| LK471-AN | LK461-AN | Norway |
| LK471-AP | LK461-AP | France |
| LK471-AS | LK461-AS | Spain |
| LK471-AV | LK461-AV | Portugal |
| LK471-AQ | LK461-AQ | Canada (English) |
| LK471-AC | LK461-AC | Canada (French) |
|  | LK461-AL | Switzerland (German) |
|  | LK461-AM | Sweden |

Mouse and Extension Cable Kit

| PBXWS-AA | 3-button mouse (included with all systems, order as spare or replacement) |
| --- | --- |
| 2T-450KM-AA | Extension cable kit for VGA, PC style keyboard, and mouse, for use with Rackmount systems. |

Step 10—Power Cords, Keyboards, and Documentation (continued)

Pedestal Documentation: Customer and Service Kit

| Customer Kit | Service Kit |  |
| --- | --- | --- |
| QZ-00MAA-GZ | QA-00MAB-GZ | Customer and Service kit for AlphaServer 1000 4/266—English |
| QZ-00MPA-GZ | QA-00MPB-GZ | Customer and Service kit for AlphaServer 1000 4/266—French |
| QZ-00MSA-GA | QZ-00MSB-GZ | Customer and Service kit for AlphaServer 1000 4/266—Spanish |
| QZ-00MJA-GZ | QZ-00MJB-GZ | Customer and Service kit for AlphaServer 1000 4/266—Japanese |
| QZ-00MUA-GZ | QZ-00MUB-GZ | Customer and Service kit for AlphaServer 1000 4/266—Italian |
| QZ-00MGA-GZ | QZ-00MGB-GZ | Customer and Service kit for AlphaServer 1000 4/266—German |
| AG-Q95HA-BE |  | Customer and Service kit On-line documentation—English |

Step 11—Cabinet Enclosure

Select cabinet enclosure for Packaged and Base AlphaServer 1000 Rackmount systems, if required.

- H9A10 19-inch EIA Cabinet Enclosure Dimensions

- Outside:  66.9-inches high, 23.62-inches wide, 33.8-inches deep

- Internal usable rackmountable space: 56-inches high, 19-inches wide, 32.2-inches deep

| H9A10-CE | 120V Retma Cabinet assembly with dual power controller |
| --- | --- |
| H9A10-CJ | 240V Retma Cabinet assembly with dual power controller |
| H9A10-CG | 120V Retma Cabinet assembly with dual power controller and front door |
| H9A10-CK | 240V Retma Cabinet assembly with dual power controller and front door |
| H9A10-AB | Retma Cabinet assembly with no power controller |
| H9A10-AD | Retma Cabinet assembly with front door, with no power controller |

Step 12—Hardware and Software Supplemental Support Services

Hardware—Americas and Asia Pacific only

- Systems include three-year hardware warranty, on-site with 5 x 9, 24-hour response time.

- Select optional Hardware Supplemental Support Services if required.

| FM-MK4HR-36 | 5 x 9, 4-hour response time |
| --- | --- |
| FM-MK512-36 | 5 x 12, 4-hour response time |
| FM-MK616-36 | 6 x 16, 4-hour response time |
| FM-MK724-36 | 7 x 24, 4-hour response time |
| FM-MKXHW-60 | Years 1-5, next day, Onsite |
| FM-MK4HR-60 | Years 1-5, 5x9, 4-hour response time |
| FM-MK512-60 | Years 1-5, 5x12, 4-hour response time |
| FM-MK616-60 | Years 1-5, 6x16, 4-hour response time |
| FM-MK724-60 | Years 1-5, 7x24, 4-hour response time |

Step 12—Hardware and Software Supplemental Support Services (continued)

Software—Americas and Asia Pacific only

- Systems include 90-day Conformance to SPD and Telephone Advisory Support. Select optional Software Supplemental Support Services, if required.

- Software service upgrades for Windows NT include advisory and remedial software support for the time period indicated.

- Software service upgrades for DIGITAL UNIX and OpenVMS include advisory and remedial software support with new version license rights for operating system and DIGITAL NAS 200 for the time period indicated.

| FM-NTS03-12 | 12-month Software Supplemental Support for Windows NT AlphaServer 1000 4/266 systems |
| --- | --- |
| FM-NTS03-36 | 36-month Software Supplemental Support for Windows NT AlphaServer 1000 4/266 systems |
| FM-NYS03-60 | 60-month Software Supplemental Support for Windows NT AlphaServer 1000 4/266 systems |
| FM-MKOSF-12 | 12-month Software Supplemental Support for DIGITAL UNIX AlphaServer 1000 4/266 systems |
| FM-MKOSF-36 | 36-month Software Supplemental Support for DIGITAL UNIX AlphaServer 1000 4/266 systems |
| FM-MKOSF-60 | 60-month Software Supplemental Support for DIGITAL UNIX AlphaServer 1000 4/266 systems |
| FM-MKVMS-12 | 12-month Software Supplemental Support for OpenVMS AlphaServer 1000 4/266 systems |
| FM-MKVMS-36 | 36-month Software Supplemental Support for OpenVMS AlphaServer 1000 4/266 systems |
| FM-MKVMS-60 | 60-month Software Supplemental Support for OpenVMS AlphaServer 1000 4/266 systems |

Step 12a—Hardware and Software Supplemental Support Services (Europe only)

Europe does not have specific part numbers for Hardware and Software Supplemental Support Services. Prices can be quoted using the Excelerator tool; contact MCS Sales in your country for information on Hardware and Software Supplemental Support Services.

Specifications

| Shipping Dimensions | Pedestal | Rackmount |
| --- | --- | --- |
| Height Width1 Depth1 Weight | 60 cm (23.8 in.) 43 cm (16.9 in) 65 cm (25.6 in) 43 kg (95 lb) typical 71 kg (156 lb) maximum | 26.7 cm (10.5 in.) 48.2 cm (19.0 in.) 73.7 cm (29.0 in.) system chassis —& rackmount hardware 63.5 cm (25.0 in.) system chassis 29 kg (65 lb) typical |
| Installed Dimensions |  |  |
| Height Width Depth Weight | 44.2 cm (18.1 in.) 35.8 cm (14.1 in.) 53.3 cm (23 in.) 39 kg (86 lb) typical 51 kg (113 lb) maximum |  |
| Clearances |  | Service clearance |
| Front Rear Sides | 75 cm (29.5 in.) 15 cm (6 in.) None | 124.04 cm (50.0 in.) 61.00 cm (24.0 in.) None |
| Environmental | Pedestal | Rackmount |
| Temperature —Operating2 —Nonoperating —Storage (60 days) —Rate of change | 10-40° C (50-104° F) Not tested -40-66° C (-40-151° F) 11° C/hr (20° F/hr) | 10-35° C (50-95° F) Not tested -40-66° C (-40-151° F) 11° C/hr (20° F/hr) |
| Relative humidity —Operating —Nonoperating —Storage (60 days) —Rate of change | 20–80% 20–80% 10–95% 20%/hr | 20–80% 20–80% 10–95% 20%/hr |
| Maximum wet bulb temperature operating Storage (60 days) | 28° C (82° F) 39° C (115°F) |  |
| Minimum dew point temperature operating | 12° C (36° F) |  |
| Maximum heat dissipation Single supply Dual supply | Current 2390 Btu/hr 4097 Btu/hr |  |
| Air flow and quality —Intake location —Exhaust location | Front Rear | Front Rear |
| Altitude —Operating —Nonoperating | 2000 m (6562 ft) 3600 m (12,000 ft) | 2000 m (6562 ft) 3600 m (12,000 ft) |
| Mechanical shock —Operating —Nonoperating | 7.5 G 10 ms 20 G peak 30 ms | 20 G peak 30 ms |
| Vibration —Operating | 10-500 Hz .1 G peak | 10-500 Hz .1 G peak |
| Acoustics Average Declared —Operating —Idle | 6.2 LwA, B 6.5 LwAd, B 6.0 LwA, B 6.3 LwAd, B | 6.2 LwA, B 6.5 LwAd, B 6.0 LwA, B 6.3 LwAd, B |

1	Dimensions of shipping pallet; fork-lift access is on the width dimension.

2	Maximum operating temperature at Sea Level. Reduce by 1 C (1.8 F) for each 600 m (2000 ft) above Sea Level.

3		Higher altitudes are possible if maximum operating temperature is reduced (see Temperature, above); other restrictions may apply, such as maximum permissible altitude for hard drive.

Specifications (continued)

| Electrical—Universal Power Supplies are 120/240 Vac |  |  |
| --- | --- | --- |
|  | Pedestal | Rackmount |
| Nominal ac voltage Operating Voltage range Power source phase Nominal frequency Frequency range Maximum inrush current RMS current at nominal – —Voltage (steady state) —Single power supply —Dual power supply | 100-120 Vac / 220-240 Vac 90-128 Vac / 180-264 Vac Single 60 Hz 59-61 Hz / 47-53 Hz 50 Amps / 50 Amps 8.0 Amps / 4.0 Amps 4.6 Amps each supply / 2.2 Amps —each supply | 120 Vac / 220-240 Vac 90-132 Vac / 180-264 Vac Single 50-60 Hz 47-53 Hz 50 Amps |
| Power cord —Type —Length —U.S. plug — —240 V plug | IEC 320 C13 190 cm (75 in.) NEMA 5-15 | 4.57 m (15 ft.) NEMA L5-30P Socket NEMA L5-30R NEMA L6-20P Socket NEMA L6-20R |
| Regulatory |  |  |
| Agency approvals | UL Listed to UL1950 (2nd edition) CSA Certified to CAN/CSA-C22.2 No. 950-M89 TUV EN 60950 VDE 0805 GS marked ZH1/618 FCC 15J Part 15 Class B Verified CE Class B Verified VCCI Class II ITE | UL Listed to UL1950 (2nd edition) CSA Certified to CAN/CSA-C22.2 No. 950-M89 TUV EN 60950 VDE 0805 GS marked ZH1/618 EMKO-TSE (74-SEC), summary of Nordic deviations FCC 15J Part 15 Verified Class A Verified CE Class A verified VCCI Class II ITE |
| Reviewed to | AS 3260 Australian Standard SS 436 14 50 Swedish Standard NZS 6661:1989 New Zealand Standard EN 60 950: 1992 European Norm IEC 950 (2nd edition) | EN 60 950/A1, JAN. 1993 European norm, January 1993 IEC950 (2nd edition) |

Specifications (continued)

Prestige 1250EXT

UPS offerings include EIA232 port for local or network monitoring.  For complete protection, UPS  products should be used with data line surge protectors and UPS monitoring software.

Pedestal UPS

| 4N-AEABF-AA | Prestige 1250EXT, 1250 VA/900W, 1 phase, 60 Hz, 120V 6-foot cord with 5-15P plug, (4) 5-15R receptacle, 9 minutes battery at full UPS load. |
| --- | --- |
| 4N-AEABF-BF | Prestige 1250EXT, 1250 VA/900W, 1 phase, 50 Hz, 200-240V in/out selectable. Uses system power cord for detachable IEC320 input connection at UPS. Includes (3) IEC320 10A output receptacles, (2) output jumpers with IEC320 connectors to system. |
| 4N-AEAEO-PA | Hot-swap Power-Pass Module for 60 Hz UPS with (7) 5-15R Surge Protected outlets. |
| 4N-AEAEO-PB | Hot-swap Power-Pass Module for 50 Hz UPS with (6) IEC320, 10A surge protected outlets. |
| 4N-AEWAR-G1 | Prestige 5 year on-site exchange warranty upgrade (U.S. only). |

Rackmount UPS

| 4N-AEABF-CA | 1.5k VA Rackmount 120V UPS. Rackmount kit included for mounting dimensions 6.17-inches high x 17-inches wide x 24-inches deep, Weight 81 lbs. |
| --- | --- |
| 4N-AEABF-BG | 1.5k VA Rackmount 240V UPS. Requires rackmount kit for dimensions 9.6-inches high x 5.6-inches wide x 15.8-inches deep, Weight 33 lbs |
| 4N-AEAEO-RA | Rackmount kit for dimensions 22-27-inches deep |
| 4N-AEAEO-RB | Rackmount kit for dimensions 28-34-inches deep |

Data Surge Protection

| 4N-GA249-AB* | Surge protector, modem connection, wall plug-in |
| --- | --- |
| 4N-GA249-CA* | Surge protector, 10BaseT connection, wall plug-in |
| 4N-GA510-BF | Surge Protector, ThinWire connection, device port |
| 4N-GA245-xx | Surge Protector, multi-port connection, din rail/rackmount |

*	Additional plug-in data modules (4N-GA240-xx) available.

Monitoring and Unattended Shutdown Software for above UPS systems only

- Include cables, media and documentation

| 4N-AEAES-AA | Single system shutdown for Windows NT |
| --- | --- |
| 4N-AEAES-BA | NW management and multi system shutdown for Windows NT* |
| 4N-AEAES-AK | Single system shutdown for DIGITAL UNIX |
| 4N-AEAES-BK | NW management and multi system shutdown for DIGITAL UNIX* |
| 4N-AEAES-EM | Single system shutdown for OpenVMS |

*	Require network adapters:  4N-AEAEO-DA/DC for 120V 60 Hz; or 4N-AEAEO-DB/DD for 220V 50 Hz
-DA and -DB = Twisted Pair; -DC and -DD = ThinWire