# 1K / 4K SROM Templates

# For SROM Format Version 4.0

Some Important Basic Rules:

- The magic block, if exists, is always located at the end of the SROM:

- In a 1K SROM it is located at bytes 96:127.

- In a 4K SROM it is located at bytes 480:511.

- In a 1K SROM:

The CRC is located at bytes 126:127 if no magic block is present, and at bytes 94:95 if a magic block is present. (As written in the spec, chapter 3 – SROM_CRC.)

In a 4K SROM:

The CRC is located at bytes 126:127 if no magic block is present, and at bytes 94:95 if a magic block is present.

If the user wants to add media, so that the info leaves size exceed byte #93, then the CRC will be in bytes 126:127, even though there is a magic block (since the magic block is at the end of the 4K).

The next rule concludes from the previous one:

- In a 4K SROM, when the CRC is located in bytes 126:127, we are forbidden to conclude that a magic block does not exist.

- Looking for a magic block is done by checking the magic CRC at the end of the SROM. Since in a 4K SROM with CRC in bytes 126:127, we have no other way to conclude whether there is a magic block or not, we assume that if the magic block CRC is not correct, then a magic block does not exist.

- For both sizes of SROM, we always program all the SROM, we do not program parts of it. That means for example that when programming a 1K template into a 4K SROM, the last 3K of the SROM will be programmed to zeros.

- When possible, that is, when the info leaves do not exceed byte #93, and there are no        tuples, or the tuples are not in the SROM, we will save the template as a 1K. In a case that the original template was of 4K and the info leaves were shorten, and/or the tuples were moved from the SROM, we shrink the template to a 1K (after verifying with the user).

- If    the user has, a legal template of 1K size that has offset to tuples in the SROM

(note that this case is possible only if that template was created using the previous

version of SROMUTIL, version 4.02), it will be enlarged to a 4K template, and will

be useful for future use only for 4K SROMs.

- SROMUTIL    supports multiple device, but if the user creates a template with more than one device:

- We do not support magic mode (even if the devices are totally identical), since it is not good for more than one device to have the same magic address.

- We support ID block only if the devices are totally identical (also identical revisions), since if they are not identical, they cannot share the same SubSystemID, CIS pointer and modem information.

A. Creating a Template Using SROMUTIL

After the user has entered the NIC’s data via SROMUTIL screens, the utility creates a template according to the following rules:

- If the info leaves size does not exceed byte #93:

- If there is no magic block, we calculate CRC and put it in bytes 126:127.

If there are no tuples, or if the tuples are not in the SROM, the template will be of a

1K size.

If, on the other hand, there are tuples in the SROM, the template will be of 4K.

- If a magic block exists, we calculate CRC and put it in bytes 94:95.

If there are no tuples, or if the tuples are not in the SROM, the template will be of a

1K size.

If, on the other hand, there are tuples in the SROM, the template will be of 4K.

- If the info leaves size exceeds byte #93:

- If there is no magic block:

Calculate CRC on bytes 0:125 and put it in bytes 126:127.

If there are no tuples, or if the tuples are not in the SROM, the template will be of a

1K size.

If, on the other hand, there are tuples in the SROM, the template will be of 4K.

- If there is a magic block:

The user has two options:

Delete some leaves so they won’t exceed byte #93,

or agree to save the template as a template of 4K size that will be good for use only

with a 4K SROM. In the second case, the CRC will be calculated on bytes 0:125 and

will be put in bytes 126:127. The magic block will be put at the end of the 4K.

If the first case, where the user agrees to delete leaves, but there are tuples located in

the SROM, the template will be saved as a 4K.

## B. Reading a Template / an SROM

There is no difference between reading a template and reading an SROM.

If a template file is read, then it is known if it is a 1K or a 4K template.

If an SROM is read, we know when reading it if it is a 1K (six bits address), or a 4K (eight bits address) SROM.

When reading a template/an SROM, either of 1K or of 4K size, we always check the CRC in bytes 126:127 first and only if it is not correct, we check the CRC in bytes 94:95.

### Reading a 1K template/SROM

- If the CRC in bytes 126:127 is correct, then there is no magic block.

If the user wants to add a magic block, there are two cases:

- Info leaves size does not exceed byte #93:

The user can add a magic block and then a CRC will be calculate for bytes 0:93 and

will be put in bytes 94:95 and the magic block with its CRC will be put in bytes

96:127. The template will be of 1K size.

- Info leaves size exceeds byte #93:

The user has two options for adding a magic block:

Delete some info leaves,

or save the template a 4K one.

- If the CRC in bytes 126:127 is not correct, then we check the CRC in bytes 94:95.

- If the CRC in bytes 94:95 is correct, than we check the magic block CRC.

- If the magic block CRC is correct, then we have a legal 1K template/SROM with a magic block.

- If the magic block CRC is not correct, then we have an illegal template/SROM, because when the CRC is in bytes 94:95, then there must be a magic block. We do not handle this template/SROM.

- If also the CRC in bytes 94:95 is not correct, then this is an illegal template/SROM, and we do not handle it.

### Reading a 4K template/SROM

- If the CRC in bytes 126:127 is correct, then we look for a magic block at the end of the 4K:

- If the magic block CRC is correct, then we have a legal 4K template/SROM with a magic block.

- If the magic block CRC at the end of the 4K is not correct, then we have a legal 4K template/SROM with no magic block (for a more detailed explanation, look at sections 2 and 3 at the beginning of the document).

- If the CRC at bytes 126:127 is not correct, then we check the CRC in bytes 94:95.

- If the CRC in bytes 94:95 is correct, then we look for a magic block at the end of the 4K:

- If the magic block CRC is correct, than we have a legal 4K template/SROM with a magic block.

- If the magic block CRC is not correct, than we have an illegal 4K template/SROM, and we do not handle it.

- If also the CRC in bytes 94:95 is not correct then this is an illegal template/SROM, and we do not handle it.

## C. Programming an SROM

As a rule, when programming an SROM, we do not change the template.

We do not add or delete a magic block, or delete info leaves, etc.

Things like that are done only via SROMUTIL. The SROMPROD does not change the template, and if a template does not feet into an SROM, we do not program. We try as much as we can to take a template and program it, as it is, into an SROM.

- Programming a 1K SROM using a 1K template with/without a magic block:

No problem.

- Programming a 4K SROM using a 4K template with/without a magic block:

No problem.

- Programming a 4K SROM using a 1K template without a magic block:

We put the 1K into the first 1K of the 4K SROM as it is. The last 3K of the SROM

are programmed to zero.

- Programming a 4K SROM using a 1K template with a magic block:

The first 96 bytes of the 1K template are programmed into the first 96 bytes of the

4K SROM, bytes 96:479 are programmed to zeros. Bytes 96:127 in the 1K template,

which contain the magic block, are programmed into bytes 480:511 of the 4K SROM.

- Programming a 1K SROM using a 4K template: Impossible.

This is because when creating the template, it was created in a 4K format, since the data wouldn’t fit into 1K. Thus, the 4K template won’t fit into a 1K SROM.

##### Appendix A

There are some things we do to avoid confusion and misunderstandings:

- When the user creates a template, by default it is a 1K template.

The only ways to make this a 4K template are:

- When the user adds media so that the info leaves size exceeds byte #93, and he also wants a magic block to be present, he has an option to save this template as a 4K one (only for devices that support magic block, of course).

- When the user wants the tuples to be located in the SROM.

- We let the user add tuples only to devices that support CardBUS.

- When we program a 4K SROM we program bytes 128:511 (or bytes 128:479, in case there is a magic block) to zeros. This will prevent many mistakes, for example: programming a 4K SROM that has tuples in it, with a 4K SROM that has tuples in a different location. If we do not program the last 3K to zeros, there will be a mismatch between the CIS pointer and the old tuples that remained in their old place.